# Definition of Readiness Level (TRL/ASRL) -REFERENCE

| Level | TRL Technology Readiness Level (Space and Ground Segments) |  |  |  | ASRL Application Service Readiness Level (System and Application) |
| --- | --- | --- | --- | --- | --- |
|  | Capabilities | Space Segment model | Ground Segment model | Software model |  |
| 1 | Basic principles observed and reported |  | Idea or concept | Research results or preliminary algorithm | not applicable |
| 2 | Technology concept/ application formulated |  | Concept supported by paper | Individual algorithms for main functions | Application/service concept formulated, market opportunities not yet addressed |
| 3 | Analytical and experimental critical function or characteristic proof-of-concept | Mathematical models, supported e.g. by sample tests | Demonstrate feasibility | Prototype of the main functions | Concept analysis performed and target market identified |
| 4 | Component and/or breadboard functional verification in laboratory environment | Breadboard | Partial prototype | Alpha version covering the main functions | Application/service verification in laboratory environment, market segment(s) and customers/users identified |
| 5 | (Critical function of) component / breadboard verified in a relevant environment | Scaled EM for the critical functions | Reduced scale prototype (for large pieces) | Beta version covering all functions | Application/service verified using operational elements, customers/users not involved |
| 6 | Demonstration of element critical functions in a relevant environment | Full scale EM representative for critical functions | Full prototype to demonstrate functionality | Product | Demonstration of prototype in relevant environment, price policy identified |
| 7 | Demonstration of element performance in the operational environment | QM/EQM/PFMa | Verified Product with final BOM, layouts, released software, full GUI | Integrated product validated in a pilot case (trial) | Pilot (Trials) with customers/users to validate utilisation and business models |
| 8 | Actual system completed and accepted for flight | PFM/FM | Validated Product in operation and commercial offer ready | Integrated product validated for full operation | Application/service completed and validated, commercial offer ready |
| 9 | “Flight proven” system through successful mission operations | PFM/FM | Product operationally deployed and used by paying customer | Live product validated in a mission | Application/service operationally deployed and used by paying customers |

a 	A PFM may be used to achieve qualification provided that the commercial customer accepts the risk and it is demonstrated that the use of an alternative qualification model (e.g. EQM) is not viable. In this case the cost of the flight hardware is not supported by ESA